TAP126- 5: Problems on capacitors

Electron charge = -1.6 x 10-19 C

1. 	What is meant by the capacitance of a capacitor?

2. 	Define the farad.

3. (a) 	A capacitor of capacitance 5 F is connected to a 6 V supply. What charge is stored in the capacitor?

(b) 	A 400 pF capacitor carries a charge of 2.5 x 10-8 C. What is the potential difference across the plates of the capacitor?

C

4.5 V

4. 	A capacitor is charged such that there is a charge of +20 mC on the positive plate. What 	is the charge on the negative plate?

5. 	A 4700 F capacitor is connected as shown in the circuit diagram. When it is fully charged:

(a) 	what is the charge on the positive plate of the capacitor?

(b) 	what is the potential difference across the capacitor?

(c) 	how many additional electrons are on the negative plate?

C

4.5 V

6. 	A resistor of 100 Ω is now added to the circuit as shown in the second diagram.

(a) 	What effect does this have on the time to charge up the capacitor?

(b) 	What is the final charge on the plates?

(c) 	What is the final potential difference across the capacitor?