TAP 416-6: Transformers

This arrangement is an experiment to investigate electromagnetic induction.

State, giving a reason for your answer in each case, what is observed as:

1.	The switch S is closed.

2.	Switch S remains closed.

3.	Switch S is reopened.

The battery is replaced by an ac supply. The voltmeter can register either dc or ac

4.	State what will be observed with switch S closed.

5.	Give a reason for your answer.

6.	Door bells in houses are often connected to the mains electricity supply through a step-down transformer. Here are two circuits to do this.

A

B

Which circuit do you consider to be less expensive to operate once it has been installed? Explain your choice.

7.	In petrol engines, the fuel is ignited by a spark across a gap which must be less than about 0.60 mm. To establish the field necessary for this spark, voltages of up to 40 kV are needed, using only a 12 V battery. To achieve this, two coils are wound around the same iron core.

The secondary coil is in series with the spark gap. The primary coil is in series with the battery and a contact breaker. When the primary circuit is broken, a spark is produced. The capacitor prevents sparking across the contact breaker.

Explain why a large voltage is induced when the contact with the battery is broken.

8	Many home electrical devices, such as televisions, refrigerators and hi-fi systems, 	incorporate transformers to step down the supply voltage.

(a) 	An ideal transformer with a primary coil of 3000 turns provides a 9 V ac output when the input is 230 V ac. How many turns must there be on the secondary coil?

(b) 	No transformer is 100% efficient, although most are nearly so. State two effects that may in practice reduce the efficiency of a transformer.

(c) 	Explain why a transformer cannot provide an output of 20 V dc when connected to a car battery whose emf is 12 V.